TD53C20 Wiring Manual

Ver 1.0

Contents

1. Main Interface of TD53C20	3

1.1 TD53C20 Interface Diagram	3

1.2 Pin Definition of TD53C20 Interface Board’s Functional Interfaces	4

2. Appendix: Basic Connection	6

2.1 DIP1 Power Port	6

2.2 DIP1 Network Interface	6

2.3 DIP2 light board control signal input and output interface	7

2.4 DIP9/DIP11 Motor Interface	8

# 1. Main Interface of TD53C20

## 1.1 TD53C20 Interface Diagram

Veneer behind Interface Figure           Veneer front Interface Figure

## 1.2 Pin Definition of TD53C20 Interface Board’s Functional Interfaces

DIP1：Power input and network interface

| 1 | DC12V |
| --- | --- |
| 2 | GND |
| 3 | Rx- |
| 4 | Rx+ |
| 5 | Tx- |
| 6 | Tx+ |
| 7 | RST |
| 8 | ACT |

DIP2：light board control signal input and output interface

| 1 | Infrared near light lamp |
| --- | --- |
| 2 | Infrared high beam |
| 3 | White near light lamp |
| 4 | White high beam |
| 5 | GND |
| 6 | Alarm IO |

DIP3：IR-CUT Interface

| 1 | IR-CUT connection port 1 |
| --- | --- |
| 2 | IR-CUT connection port 2 |

DIP4：WIFI antenna interface

Connect the WIFI antenna

DIP5：Audio output interface

Recommended horn 8Ω/2W

| 1 | Speaker positive |
| --- | --- |
| 2 | Speaker Negative |

DIP6：Audio input interface

| 1 | MIC positive |
| --- | --- |
| 2 | MIC Negative |

DIP7：T-flash card Socket

flash card Socket

DIP8：Reset key

Support Plate end reset

DIP9：Vertical motor control interface

| 1 | 1A |
| --- | --- |
| 2 | 1B |
| 3 | 2A |
| 4 | 2B |

DIP10：IR-CUT Interface

| 1 | IR-CUT connection port 1 |
| --- | --- |
| 2 | IR-CUT connection port 2 |

DIP11：Horizontal motor control interface

| 1 | 1A |
| --- | --- |
| 2 | 1B |
| 3 | 2A |
| 4 | 2B |

# 2. Appendix: Basic Connection

## 2.1 DIP1 Power Port

12V

GND

12V/2A

Power Supply

1

DIP1

2

## 2.2 DIP1 Network Interface

Example 1: Method of directly connect to network cable

Green-white

Pick-up RST Reset Button

3  4  5  6   7   8

Orange

Orange-white

Green

6

3

1

2

GND

Example 2: Lead up to an RJ45 seat, then use the method of cross-wire and PC connection

1.TX+

2.TX-

3.RX+

4.NU

5.NU

6.RX-

7.NU

8.NU

RJ45 Pin

3  4  5  6  7  8

DIP2

Cross line

2.3 DIP2 light board control signal input and output interface

1

2

3

4

5

6

Infrared near light lamp

Infrared high beam

White near light lamp

White high beam

GND

Alarm IO

## 2.4 DIP9/DIP11 Motor Interface

1）Horizontal/Vertical motor specifications

| Serial number | Project | Specifications |
| --- | --- | --- |
| 1 | Voltage | 5V DC |
| 2 | Number of phases | 4 |
| 3 | Slowdown ratio | 1:64 |