TD53J20 Wiring Manual

Ver 1.0

Contents

1. Main Interface of TD53J20	3

1.1 TD53J20 Interface Diagram	3

1.2 Pin Definition of TD53J20 Interface Board’s Functional Interfaces	4

2. Appendix: Basic Connection	6

2.1 DIP1 Power Port	6

2.2 DIP1 Network Interface	6

2.3 DIP10 light board control signal input and output interface	7

2.4 DIP6/DIP7 Motor Interface	8

# 1. Main Interface of TD53J20

## 1.1 TD53J20 Interface Diagram

Veneer behind Interface Figure           Veneer front Interface Figure

## 1.2 Pin Definition of TD53J20 Interface Board’s Functional Interfaces

DIP1：Power input and network interface

| 1 | DC12V |
| --- | --- |
| 2 | GND |
| 3 | Rx- |
| 4 | Rx+ |
| 5 | Tx- |
| 6 | Tx+ |
| 7 | RST |
| 8 | ACT |

DIP2：WIFI antenna interface

Connect the WIFI antenna

DIP3：Audio output interface

Recommended horn 8Ω/2W

| 1 | Speaker positive |
| --- | --- |
| 2 | Speaker Negative |

DIP4：T-flash card Socket

flash card Socket

DIP5：Reset key

Support Plate end reset

DIP6：Vertical motor control interface

| 1 | 4phase |
| --- | --- |
| 2 | 3phase |
| 3 | 2phase |
| 4 | 1phase |
| 5 | 12V |

DIP7：Horizontal motor control interface

| 1 | 4phase |
| --- | --- |
| 2 | 3phase |
| 3 | 2phase |
| 4 | 1phase |
| 5 | 12V |

DIP8：IR-CUT Interface

| 1 | IR-CUT connection port 1 |
| --- | --- |
| 2 | IR-CUT connection port 2 |

DIP9：Audio input interface

| 1 | Audio input signal |
| --- | --- |
| 2 | GND |

DIP10：light board control signal input and output interface

| 1 | Alarm IO |
| --- | --- |
| 2 | Positive pole of Infrared light lamp |
| 3 | Positive pole of white light lamp |
| 4 | Infrared / white anode |

# 2. Appendix: Basic Connection

## 2.1 DIP1 Power Port

12V

GND

12V/2A

Power Supply

1

DIP1

2

## 2.2 DIP1 Network Interface

Example 1: Method of directly connect to network cable

Green-white

Pick-up RST Reset Button

3  4  5  6   7   8

Orange

Orange-white

Green

6

3

1

2

GND

Example 2: Lead up to an RJ45 seat, then use the method of cross-wire and PC connection

1.TX+

2.TX-

3.RX+

4.NU

5.NU

6.RX-

7.NU

8.NU

RJ45 Pin

3  4  5  6  7  8

DIP2

Cross line

2.3 DIP10 light board control signal input and output interface

1

2

3

4

Alarm IO

Positive pole of Infrared light lamp

Positive pole of white light lamp

Infrared / white anode

## 2.4 DIP6/DIP7 Motor Interface

Horizontal/Vertical motor specifications

| Serial number | Project | Specifications |
| --- | --- | --- |
| 1 | Voltage | 12 V DC |
| 2 | Number of phases | 4 |
| 3 | Slowdown ratio | 1:25.4359 |